Absolutely — I can write a safe, step-by-step Phase 1 → Phase 8 for Debian Bookworm Lite, with special care for Phase 2–4 so you don’t lose SSH. I’ll include verification steps and kernel forwarding for internet sharing.

Here’s the full sequence:

Absolutely — here’s a clean, step-by-step recap of everything you’ve done so far for eth0 static IP (recovery path) on Raspberry Pi OS Bookworm Lite using NetworkManager.

This is your Phase 1: Secure eth0 with a static IP (192.168.50.2/24).

🧩 Step 1 — Confirm NetworkManager is in control

Check that Bookworm is using NetworkManager (not dhcpcd):

systemctl status NetworkManager

systemctl status dhcpcd

Expected:

- NetworkManager → active (running)

- dhcpcd → not found / inactive

➡️ Conclusion: We must use nmcli, not /etc/dhcpcd.conf.

🧩 Step 2 — List existing network connections

See all NetworkManager profiles:

nmcli con show

You identified:

- preconfigured → Wi-Fi on wlan0

- Wired connection 1 → Ethernet profile for eth0 (DEVICE -- since no cable)

🧩 Step 3 — Force eth0 into manual (static IP) mode

Tell NetworkManager:

“Whenever eth0 is up, always use 192.168.50.2/24 — no DHCP.”

Command:

sudo nmcli con modify "Wired connection 1" \

ipv4.method manual \

ipv4.addresses 192.168.50.2/24 \

ipv4.gateway "" \

ipv4.dns ""

What this does:

- ipv4.method manual → disables DHCP

- ipv4.addresses 192.168.50.2/24 → sets static IP

- Clears gateway + DNS (good for a direct PC↔Pi link)

🧩 Step 4 — Apply the profile (even if cable isn’t plugged)

sudo nmcli con down "Wired connection 1"

sudo nmcli con up "Wired connection 1"

You saw errors for down because it wasn’t active — that’s normal.
up succeeded and saved the profile.

🧩 Step 5 — Verify static IP is stored in config

Check the profile:

nmcli con show "Wired connection 1" | grep ipv4

Expected:

ipv4.method:    manual

ipv4.addresses: 192.168.50.2/24

➡️ Confirms the static IP is persisted on disk.

🧩 Step 6 — Check interface state (runtime)

ip addr show eth0

If no cable plugged in, you’ll see:

NO-CARRIER ... state DOWN

(no inet line)

If cable plugged in, you’ll see:

inet 192.168.50.2/24 scope global eth0

🧩 Step 7 — Reboot test

sudo reboot

After reboot:

- Profile still shows 192.168.50.2/24 ✔️

- IP only appears on eth0 when a cable is plugged ✔️

🎯 Final Result

You now have:

🛟 A permanent recovery interface
• eth0 → 192.168.50.2/24
• Survives reboot
• Independent of Wi-Fi / AP

SSH into rpi using eth0 before moving on to phase 2

ssh rpi@192.168.50.2

When you’re ready, next is:

➡️ Phase 2 — Turn wlan0 into a Wi-Fi Access Point (safely)
No more accidental SSH lockouts 😄

Just say:
👉 “Start Phase 2”

Absolutely — here’s a clean, full setup sequence for turning your Raspberry Pi (Bookworm Lite) into a Wi-Fi access point with:

- SSID: MDP_Grp22

- Password: mdp12345 (valid 8-char WPA2 passphrase)

- Static IP: 192.168.22.1/24

- Hostapd + dnsmasq + nftables configured

- Auto-start on reboot

- Leaves the existing Wi-Fi client profile (preconfigured) intact

Phase 1: Prepare NetworkManager and wlan0

- Make wlan0 unmanaged by NetworkManager (so hostapd can control it):

sudo nano /etc/NetworkManager/conf.d/unmanaged.conf

Add:

[keyfile]

unmanaged-devices=interface-name:wlan0

Restart NM:

sudo systemctl restart NetworkManager

Phase 2: Configure the AP connection via nmcli

- Add a new Wi-Fi connection for AP mode:

sudo nmcli con add type wifi ifname wlan0 con-name ap-wlan0 autoconnect yes ssid MDP_Grp22

- Set it to AP mode with manual IP:

sudo nmcli con modify ap-wlan0 \

802-11-wireless.mode ap \

802-11-wireless.band bg \

ipv4.method manual \

ipv4.addresses 192.168.22.1/24 \

ipv4.gateway "" \

ipv4.dns ""

Phase 3: Configure hostapd

- Create /etc/hostapd/hostapd.conf:

sudo nano /etc/hostapd/hostapd.conf

Paste:

interface=wlan0

driver=nl80211

ssid=MDP_Grp22

hw_mode=g

channel=6

wmm_enabled=1

macaddr_acl=0

auth_algs=1

ignore_broadcast_ssid=0

wpa=2

wpa_passphrase=mdp12345

wpa_key_mgmt=WPA-PSK

rsn_pairwise=CCMP

- Point hostapd default to this config:

sudo nano /etc/default/hostapd

Uncomment/set:

DAEMON_CONF="/etc/hostapd/hostapd.conf"

Phase 4: Configure dnsmasq (DHCP server for AP clients)

- Backup default:

sudo mv /etc/dnsmasq.conf /etc/dnsmasq.conf.backup

- Create new config:

sudo nano /etc/dnsmasq.conf

Paste:

interface=wlan0

dhcp-range=192.168.22.10,192.168.22.50,255.255.255.0,24h

domain-needed

bogus-priv

Phase 5: Enable IP forwarding & NAT (nftables)

- Enable IP forwarding:

sudo nano /etc/sysctl.conf

Uncomment/set:

net.ipv4.ip_forward=1

Apply:

sudo sysctl -p

- Setup nftables:

sudo nano /etc/nftables.conf

Paste:

table ip nat {

chain prerouting {

type nat hook prerouting priority dstnat; policy accept;

}

chain postrouting {

type nat hook postrouting priority srcnat; policy accept;

oifname "eth0" masquerade

}

}

table inet filter {

chain input {

type filter hook input priority filter; policy accept;

}

chain forward {

type filter hook forward priority filter; policy drop;

iifname "wlan0" oifname "eth0" accept

iifname "eth0" oifname "wlan0" ct state established,related accept

}

chain output {

type filter hook output priority filter; policy accept;

}

}

Enable/start nftables:

sudo systemctl enable nftables

sudo systemctl start nftables

sudo nft list ruleset  # verify rules

Phase 6: Enable and start services

sudo systemctl unmask hostapd

sudo systemctl enable hostapd

sudo systemctl enable dnsmasq

sudo systemctl restart hostapd

sudo systemctl restart dnsmasq

Phase 7: Test your AP

- Make sure wlan0 is up:

sudo ip link set wlan0 up

ip link show wlan0

- state UP is what you want (NO-CARRIER is fine).

- Check hostapd logs if needed:

sudo journalctl -u hostapd -f

- Connect a device to SSID MDP_Grp22 with password mdp12345.

- Your Pi should provide IPs in the range 192.168.22.10–50.

✅ This setup keeps your original preconfigured Wi-Fi client connection intact, so your Pi can still connect to home Wi-Fi when not acting as an AP.

If you want, I can also give a shortcut “one-shot script” that sets all this up automatically — no manual editing needed, just copy-paste.

Do you want me to make that?

Perfect — you can switch eth0 back to DHCP without touching your AP setup. Here’s the step-by-step with NetworkManager (nmcli):

1️⃣ Check your current connections

nmcli con show

You should see something like:

NAME                UUID                                  TYPE      DEVICE

Wired connection 1  4bd17126-fe6a-3b62-8999-eca21e9c372f  ethernet  eth0

ap-wlan0            ...                                   wifi      wlan0

Wired connection 1 is your eth0 profile.

2️⃣ Change eth0 to DHCP

sudo nmcli con modify "Wired connection 1" ipv4.method auto

sudo nmcli con modify "Wired connection 1" ipv4.gateway ""

sudo nmcli con modify "Wired connection 1" ipv4.dns ""

- ipv4.method auto → tells NetworkManager to request DHCP

- Clear gateway & DNS because DHCP will provide them

3️⃣ Restart eth0 connection

sudo nmcli con down "Wired connection 1"

sudo nmcli con up "Wired connection 1"

- nmcli con up will request a DHCP lease.

- You can check the IP with:

ip addr show eth0

You should see a dynamic IP assigned by your router and your default gateway automatically set.

4️⃣ Test internet

ping -c 3 8.8.8.8

ping -c 3 google.com

- If both work, your AP clients should now have internet access via eth0 + NAT.

💡 Tip: This keeps your AP IP on wlan0 (192.168.22.1) unchanged, so you don’t break your AP setup.

If you want, I can write a mini “switch AP + eth0 to internet” command sequence so you can toggle easily between static eth0 (for offline AP) and DHCP eth0 (for internet).

Do you want me to do that?

2️⃣ Bluetooth SPP (Classic Bluetooth → Android Serial)

🔹 Step 1 — Install tools

sudo apt install -y bluez bluetooth rfkill

🔹 Step 2 — Enable Classic compatibility

sudo nano /etc/systemd/system/dbus-org.bluez.service

Find:

ExecStart=/usr/lib/bluetooth/bluetoothd

Change to:

ExecStart=/usr/lib/bluetooth/bluetoothd -C

Reload & reboot:

sudo systemctl daemon-reexec

sudo reboot

🔹 Step 3 — Pair with Android

On Pi:

bluetoothctl

Inside bluetoothctl:

power on

agent on

default-agent

discoverable on

pairable on

scan on

On Android:
• Turn on Bluetooth
• Find raspberrypi → Pair

Then on Pi:

pair XX:XX:XX:XX:XX:XX

trust XX:XX:XX:XX:XX:XX

connect XX:XX:XX:XX:XX:XX

quit

🔹 Step 4 — Setup SPP

sudo sdptool add --channel=1 SP

sudo rfcomm listen /dev/rfcomm0 1

Test:

echo "Hello from Pi" > /dev/rfcomm0

cat /dev/rfcomm0

Use Serial Bluetooth Terminal app on Android.

3️⃣ USB Interface (Pi ↔ STM32F)

When STM32F is plugged in via USB:

ls /dev/ttyACM*

ls /dev/ttyUSB*

You’ll see something like:

/dev/ttyACM0

Test serial:

sudo apt install minicom

minicom -b 115200 -D /dev/ttyACM0

Python example:

import serial

ser = serial.Serial('/dev/ttyACM0', 115200, timeout=1)

ser.write(b'Hello STM32\n')

print(ser.readline())

🧠 Summary

| Feature | Bookworm Status |
| --- | --- |
| Wi-Fi AP + Gateway | ✅ Supported via nftables |
| Bluetooth SPP | ✅ Works with bluez -C |
| USB / STM32 UART | ✅ /dev/ttyACM* |

If you want, next I can:
✔ Turn this into a 1-page checklist for your group
✔ Or give you debug commands for when something doesn’t work
✔ Or translate your Buster commands → Bookworm equivalents line-by-line